AnyVolt3

Features

3V to 24V output voltage range
5V to 30V input voltage range

3A max input or output current
<50mV typical ripple

No external components needed

1.85” x 1.05” x 1.15” (46mm x 27mm x 29mm)

Weighs 46 grams

Overcurrent protection
Thermal protection

General Description

The AnyVolt3 is a miniature step up / step down switching DC-DC converter. The output is adjustable from 3 to 24V.

The output voltage is set with a small screw potentiometer on the side of the AnyVolt3. Once the output voltage is set, it does not matter whether the input voltage is higher, lower, or the same as the desired output.

The AnyVolt3 uses four screw terminals and requires jumpers or stripped wires to operate.

Applications

Lab work and prototyping

Powering audio amplifiers

Driving multiple LEDs in series

Backlit LCD drive

Solar powered applications
Remote video transmitters

Battery powered applications

Renewable energy source applications

| Characteristic | Min | Typical | Max |
| --- | --- | --- | --- |
| Input Voltage | 5V | 5V to 30V | 30V |
| Output Voltage | 3V | 3V to 24V | 24V1 |
| Continuous Input Current |  |  | 3A |
| Continuous Output Current |  |  | 3A |
| Output Ripple (Vp-p) |  | 50mV |  |
| Quiescent current draw |  | 15mA |  |
| Efficiency – Step-down | 77% | 87% | 96% |
| Efficiency – Step-up | 82% | 85% | 89% |
| Efficiency – Buffering | 62% | 84% | 88% |
| Recommended ambient temperature range | -20ºC | 25ºC | 55ºC2 |

1Max output varies with manufacturing processes.

2Ambient temperatures higher than room temperature will decrease the amount of current AnyVolt3 can handle. For optimal performance, mount AnyVolt3 in an open space with air flowing across it.

Adjusting the output voltage

With the adjustment screw facing you, turning it clockwise increases the voltage, similar to the volume control on a stereo system.

Current limits

Input and output current to/from the AnyVolt3 should be kept track of with a multimeter if you anticipate driving heavy loads.

Stepping up from a lower to a higher voltage means that there will be a higher current on the input than the output. For this reason, ensure that both the input and output current limits of 3A aren’t being exceeded. If for some reason you cannot use a multimeter to monitor input current, you can implicitly derive the input current using the input voltage, output voltage, and output current.

Example:

The output of the AnyVolt3 in a certain project is 12V, and it is supplying a constant .99A. What will the input current be if I am powering the AnyVolt3 with a 5V supply?

“Power = Volts * Amps”

12V*.99A = 11.88W

So the power output is 11.88W.

“Output power / efficiency = Input power”

Looking at the efficiency curves of the AnyVolt3 datasheet, efficiency is 79.75% in this situation.

11.88W / 0.7975 = 14.90W

So the power going into the AnyVolt3 is 14.90W

“Power / Volts = Amps”

14.9W / 5V = 2.98A

So the current at the AnyVolt3’s input is around 2.98A. This is within limits for now, but if the input voltage supply were to drop below 5V, the overcurrent condition would be reached.

Here is a table of max loads you can power with an AnyVolt3, depending on your input voltage.

| 5V in |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 1.75 | 1.27 | .99 | .81 | .68 | .59 | .52 |
| 7V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 2.78 | 1.81 | 1.41 | 1.16 | .98 | .85 | .75 |
| 9V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 2.11 | 1.83 | 1.52 | 1.28 | 1.11 | .99 |
| 12V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 3 | 2.22 | 1.85 | 1.68 | 1.5 | 1.32 |
| 15V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 3 | 3 | 2.16 | 1.93 | 1.74 | 1.6 |
| 18V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 3 | 3 | 3 | 2.03 | 1.84 | 1.7 |
| 21V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 3 | 3 | 3 | 2.5 | 1.95 | 1.79 |
| 24V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 3 | 3 | 3 | 3 | 2.4 | 1.9 |
| 27V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 2.18 |
| 30V in |  |  |  |  |  |  |  |  |
| Output Voltage (V) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| Max current out (A) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

Overcurrent/overtemperature behavior
If the current limit has been considerably exceeded, or if the device is overheated to beyond 120ºC the AnyVolt3 will cut output current entirely. We strongly recommend practicing good engineering techniques and not overloading the device beyond operating parameters.

Additional notes

AnyVolt3 uses a 25 turn worm gear driven potentiometer and cannot wiggle loose. Do not apply glue to the voltage adjustment pot.

AnyVolt3 should be mounted at least 2 inches away from any circuitry that is sensitive to RF.

For best performance, mount AnyVolt3 in an open space with some air flowing across it to keep it cool.

Efficiency Curves