DE-SW0XX Switching Voltage Regulator

Features

Drop-in replacement for LM78XX

Up to 30V input voltage
1.3V dropout voltage

3.3V and 5V output voltages available

1A continuous output current

Efficiency up to 87%

Integrated bypass capacitors

Integrated heat spreader

Weighs only 3.8g
Can drive inductive loads

# General Description

The DE-SW0XX family of switch mode voltage regulators are designed to be the easiest possible way to add the benefits of switch-mode power to a new or existing project. The DE-SW0XX family is Pin-compatible with the common 78XX family of linear voltage regulators. They have integrated decoupling capacitors, so external capacitors are not generally necessary.  Available voltages are 3.3 and 5 volts.

The DE-SW0XX family operates over a wide input voltage range, from (Vout+1.3V) to 30v, at up to one amp of continuous output current. Efficiencies are up to 87% (Figure 2) Ripple is less than 2% of output.

The DE-SW0XX family works on a breadboard, making it an ideal solution for prototyping and one-off circuits.

Applications

Battery powered applications

Robots
Servo power
Small DC motors

Lithium battery charging and maintenance

Point of load voltage regulation

Any application where a 78XX regulator is dissipating too much heat or a large heatsink is undesirable

Typical Performance Characteristics

The device can be expected to perform as characterized within these parameters

| Characteristic | Min | Typical | Max |
| --- | --- | --- | --- |
| Input voltage | Vout+1.3V |  | 30V |
| Output Current (RMS)1 | 0A |  | 1A |
| Pulsed Output Current (5 sec) |  |  | 1.5A |
| Output Ripple | 30mV | 70mV | 100mV |
| Efficiency (See Figure 2 and Figure 3) | 65% | 83% | 87% |
| Transient response in load regulation (0-1A pulses, 1ms, Vp-p) |  | 4% |  |
| Power dissipation | 100mW | 800mW | 1.2W |
| Power output in still air | 0W |  | 5W |
| Quiescent current draw (Vin = 12V) |  | 16mA |  |
| Switching frequency | 230kHz | 270kHz | 290kHz |
| Thermal Derating in still air | See Figure 4 |  |  |

1For input voltages above 25V, an output current of at least 40mA is needed to maintain the regulated output voltage. This can be accomplished by adding a 1kΩ load resistor, or by simply connecting the load you wanted to use anyway.

## Absolute Maximum ratings

Operation beyond these parameters may permanently damage the device

| Characteristic | Min | Max |
| --- | --- | --- |
| Input voltage | 0V | 35v |
| Output Current | 0A | 1.5A |
| Power dissipation |  | 1.5W |
| Ambient Temperature | -20C | 70C |

Overcurrent/overtemperature behavior
If the current limit has been considerably exceeded or if the device is overheated the product will gradually reduce the output voltage in an attempt to reduce the load on the device. Once the extra load is removed or the temperature is brought down, the desired output voltage will be restored. It is unlikely that you will destroy the regulator by exceeding the current/temperature ratings but we still recommend practicing good engineering techniques and do not overload the device beyond the recommended operating parameters.

## Figure 2: Efficiency vs. Input Voltage

## Figure 3: Efficiency vs. Output Current

Figure 4: Thermal Derating in Still Air

## Typical Applications

General-purpose regulator

Regulator for ripple sensitive devices