DE-SWADJ Adjustable Switching Voltage Regulator

Features

Drop-in replacement for any of the LM78XX

regulators

Outputs any voltage between 1.25v and 13v

Adjustment is by a 25-turn potentiometer, making

it easy to dial in exactly the right output

3 to 30V input voltage

Up to 10W output power
1A continuous output current2

Efficiency up to 92%

Integrated bypass capacitors

Integrated heat sink

Weighs only 5.5g

# General Description

The DE-SWADJ is designed to be the easiest possible way to utilize the benefits of switch-mode power when you need an unusual or easily changed voltage. The DE-SWADJ family is pin-compatible with the common 78XX family of linear voltage regulators, and can step down to 1.25v to 13v with no external circuitry required. It has integrated decoupling capacitors, so external capacitors are not generally necessary.

The DE-SWADJ operates over a wide input voltage range, from 3v to 30v, at up to one amp of continuous output current.  Maximum power output is 10W.  Efficiencies are up to 92% (Figure 2). Ripple is less than 2% of output.

The DE-SWADJ works on a breadboard, making it an ideal solution for prototyping and one-off circuits.

Applications

Battery powered applications

Robots

Battery charging and maintenance

Point of load voltage regulation

Any application needing a nonstandard voltage

Overclocking and over or under-volting standard products for more performance or less power draw

Figure 1

Typical Performance Characteristics

The device can be expected to perform as characterized within these parameters

| Characteristic | Min | Typical | Max |
| --- | --- | --- | --- |
| Input voltage1 | 3V |  | 30V |
| Output Voltage, Min |  | 1.25v |  |
| Output Voltage, Max | 13v | 14v | 14.5v |
| Output Power |  |  | 10W |
| Output Current (RMS)2 | 0A |  | 1A |
| Pulsed Output Current (5 sec) |  |  | 1.5A |
| Output Ripple | 30mV | 70mV | 100mV |
| Efficiency (See Figure 2) | 55% |  | 92% |
| Transient response in load regulation (0-1A pulses, 1ms, Vp-p) |  | 4% |  |
| Power dissipation | 100mW | 800mW | 1.2W |
| Power output in still air | 0W |  | 10W |
| Switching frequency | 230kHz | 270kHz | 290kHz |

1 The sum of input and output voltages must not exceed 40v

2 Limited at high voltages by the max power output – see graph below

2 For input voltages above 25V, an output current of at least 40mA is needed to maintain the regulated output voltage. This can be accomplished by adding a 1kΩ load resistor, or by simply connecting the load you wanted to use anyway.

## Absolute Maximum ratings

Operation beyond these parameters may permanently damage the device

| Characteristic | Min | Max |
| --- | --- | --- |
| Input voltage | 0V | 35v |
| Output Current | 0A | 1.5A |
| Power dissipation |  | 1.5W |
| Ambient Temperature | -20C | 70C |

Overcurrent/overtemperature behavior
If the current limit has been considerably exceeded or if the device is overheated the product will gradually reduce the output voltage in an attempt to reduce the load on the device. Once the extra load is removed or the temperature is brought down, the desired output voltage will be restored. It is unlikely that you will destroy the regulator by exceeding the current/temperature ratings but we still recommend practicing good engineering techniques and do not overload the device beyond the recommended operating parameters.

Additional notes

DE-SWADJ uses a 25 turn worm gear driven potentiometer and cannot wiggle loose. Do not apply glue to the voltage adjustment pot. Turning the pot clockwise reduces the output voltage.

For best performance, mount DE-SWADJ in an open space with some air flowing across it to keep it cool.

## Figure 2: Efficiency vs. Input Voltage