# Sabertooth 2X5 Quick Start Guide

December 2006

Congratulations on your purchase of a Sabertooth 2X5 regenerative motor driver.  Sabertooth 2X5 is one of the most flexible and configurable motor drivers on the market. As a result, it must be set to the correct operating mode before use. Below is a generalized hookup diagram of a Sabertooth 2X5. On the reverse side is a chart of some of the most commonly used operating modes.

|  | ## Specs: |
| --- | --- |
|  | Input voltage: 6V-18V Output current: 5A Peak Output current: 10A Operating modes: Analog, R/C, Serial |

For full product documentation and manual, please visit http://www.dimensionengineering.com/Sabertooth2X5.htm

Sabertooth 2X5

Operating mode quick reference chart All options are set via the switches

|  | Analog control, linear, independent: a 0V to 5V analog input is connected to terminals S1 and S2. 0V is full reverse, 5V is full forward, 2.5V is stop. |
| --- | --- |
|  | Microcontroller pulses, independent linear control: An R/C servo signal is connected to terminals S1 and S2. A 1000us – 2000us pulse controls speed and direction. 1500us is stop. |
|  | Radio control, differential drive, exponential: An R/C servo signal is connected to terminals S1 and S2. The Sabertooth will autocalibrate the center and endpoints of the signal. |
|  | Simplified Serial, 38400 Baud: A TTL level 8N1 serial data stream is connected to terminal S1. Control is with single byte commands. Motor 1: 1 is full reverse, 64 is stop and 127 is full forward. Motor 2: 128 is full reverse, 192 is stop and 255 is full forward. |
|  | Packetized Serial, address 128: A TTL level 8N1 serial data stream is connected to terminal S1. Control is via a multi-byte packet. |
|  | Lithium cutoff option: When switch 3 is in the down position (in any operating mode) the Sabertooth will shut down at 3.0V per cell. This protects lithium batteries from damage. |

Sabertooth features many more operating modes and options not shown here. For the full manual, please visit http://www.dimensionengineering.com/