# SyRen 10 Quick Start Guide

April 2005

Congratulations on your purchase of a Syren 10 regenerative motor driver.  SyRen 10 is one of the most flexible and configurable motor drivers on the market. As a result, it must be set to the correct operating mode before use before use. Below is a generalized hookup diagram of a Syren 10. On the reverse side is a chart of some of the most commonly used operating modes.

|  | ## SyRen 10 |
| --- | --- |
|  | Input voltage: 6V-24V Output current: 10A Peak Output current: 15A Operating modes: Analog, R/C, Serial |

For full product documentation and manual, please visit http://www.dimensionengineering.com/SyRen10.htm

SyRen 10

Operating mode reference chart. All options are set via the DIP switches

|  | Analog bi-direction: a 0V to 5V analog input is connected to terminal S1. 0V is full reverse, 5V is full forward and 2.5V is stop |
| --- | --- |
|  | Analog single-direction: a 0V to 5V analog input is connected to terminal S1. 0V is stop and 5V is full forward. |
|  | R/C standard: An R/C servo signal is connected to terminal S1. A 1000us pulse is full reverse and a 2000us pulse is full forward. 1500us is stop. |
|  | R/C auto-calibrate: An R/C servo signal is connected to terminal S1. The SyRen will automatically detect the center and endpoints of the signal. |
|  | Simplified Serial, 38400 Baud: A TTL level 8N1 serial data stream is connected to terminal S1. Control is by single byte commands: 0 is full reverse, 128 is stop and 255 is full forward. |
|  | Packetized Serial, address 128: A TTL level 8N1 serial data stream is connected to terminal S1. Control is via a multi-byte packet. |
|  | Lithium cutoff option: When switch 3 is in the down position (in any operating mode) the SyRen will shut down at 3.0V per cell. This protects lithium batteries from damage. |

SyRen 10 features an additional 17 operating modes and options not shown here. For the full manual, please visit http://www.dimensionengineering.com/SyRen10.htm